DATA SHEET for LED

| Part No. | LUW80043-H |  |  |
| --- | --- | --- | --- |
| ###### Emitted Color |  | ###### Lens Color | ###### Chip Material |
| Ultra Super White |  | Water Clear | InGaN |

Absolute Maximum Rating of Each Segment (Ta = 25℃)

| Parameter | Symbol | Maximum Rating | Unit |
| --- | --- | --- | --- |
| Power Dissipation | PM | 120 | mW |
| Pulse Forward Current (Duty 1/10 @ 1kHz) | IFP | 100 | mA |
| Continuous Forward Current | IF | 30 | mA |
| Reverse Voltage | VR | 6 | V |
| Operation Temperature | Topr | －25℃∼85℃ | ℃ |
| Storage Temperature | Tstg | －40℃∼100℃ | ℃ |
| Soldering Temperature : 2.0mm from Body for 3 seconds at 260℃ |  |  |  |

Electron-Optical Characteristics of Each Segment (Ta = 25℃)

| Parameter | Symbol | Min. | Typ. | Max. | Unit | Test Condition |
| --- | --- | --- | --- | --- | --- | --- |
| Luminous Intensity | IV |  | 7,000 |  | mcd | IF=20 mA |
| Forward Voltage | VF |  | 3.0 | 3.5 | V | IF=20 mA |
| Reverse Current | IR |  |  | 20 | μA | VR=5V |
| Chromaticity Coordinates | x |  | 0.28 |  | nm | IF=20 mA |
|  | y |  | 0.31 |  | nm |  |
| Viewing Angle | 2θ1/2 |  | 20 |  | deg | IF=20 mA |
| Pay attension to electrostatic (ESD) |  |  |  |  |  |  |

Note :

- The luminous intensity data and λP  is survey values with the machine JF-Ⅱ, JS-2000.

- 2θ1/2 is the clip angle at which the luminous intensity half the axial luminous intensity.

Package Dimensions : 7.8 mm Round resin mold type

Note :

- All dimensions are in millimeters(mm)

- Tolerance is ±0.25mm unless otherwise noted

Typical Characteristic Curves :